| Python Code | Annotations |
| --- | --- |
| Initial Setup |  |
| import pandas as pd | Imports pandas library for potential table handling and data manipulation. |
| import numpy as np | Imports NumPy for numerical operations |
| cubic_yard_ft3 = 27 | Defines the volume of one cubic yard in cubic feet. All calculations are based on 1 cubic yard. |
| unit_weight_water = 62.4 | Standard unit weight of water in lb/ft³. Used to convert between weight and volume. |
| Water Weight Calculation |  |
| def calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_SCM_D, wc_ratio_E): | Defines a function to calculate required water weight. |
| total_cementious = cement_A + fly_ash_B + silica_fume_C + other_SCM_D | Adds all cementitious materials together. |
| water_weight_Q = total_cementious * wc_ratio_E | Multiplies total cementitious weight by water-cement ratio. |
| return water_weight_Q | Returns calculated water weight |
| Cementitious Volume Calculations |  |
| def calculate_volume_R(cement_A, sg_cement_J): | Function to compute cement volume. |
| volume_R = cement_A / (sg_cement_J * unit_weight_water) | Converts weight to volume using: Volume = Weight / (SG × 62.4). |
| return volume_R | Returns cement volume in cubic feet. |
| calculate_volume_S() calculate_volume_T() calculate_volume_U() | Same logic applies for Fly ash Silica fume Other SCM |
| Air Volume Calculation |  |
| def calculate_air_volume_V(air_content_F): | Defines air volume function. |
| volume_V = (air_content_F / 100) * cubic_yard_ft3 | Converts air percentage to actual volume in ft³. |
| return volume_V | Returns air volume |
| Water Volume Calculation |  |
| def calculate_water_volume_W(water_weight_Q): | Defines water volume function. |
| volume_W = water_weight_Q / unit_weight_water | Converts water weight to volume using unit weight of water. |
| return volume_W | Returns water volume in cubic feet |
| Total Aggregate Volume |  |
| def calculate_total_aggregate_volume_X(volume_R, volume_S, volume_T, volume_U, volume_V, volume_W): | Defines function to compute remaining aggregate volume |
| volume_X = cubic_yard_ft3 - volume_R - volume_S - volume_T - volume_U - volume_V - volume_W | Subtracts all material volumes from total 27 ft³ |
| return volume_X | Returns total aggregate volume available |
| Fine Aggregate Weight |  |
| def calculate_fine_aggregate_Y(percent_fine_G, sg_fine_N, volume_X): | Function to compute fine aggregate weight |
| weight_Y = unit_weight_water * (percent_fine_G / 100) * sg_fine_N * volume_X | Converts aggregate volume portion to weight using specific gravity |
| return weight_Y | Returns fine aggregate weight in pounds |
| calculate_coarse_aggregate_Z() calculate_other_aggregate_AA() | Same Structure applies |
| User Input Section |  |
| project_no = int(input("Enter project number: ")) | Collects project number |
| concrete_class = input("Enter class of concrete: ") | Collects concrete class. |
| cement_A = float(input("Enter weight of cement: ")) | User enters cement weight in pounds |
| wc_ratio_E = float(input("Enter water cement ratio: ")) | User enters w/c ratio |
| air_content_F = float(input("Enter target air content: ")) | User enters air content percentage. |
|  | Other inputs follow same pattern for fly ash, silica fume, aggregates, and specific gravities. |
| Execution of Calculations |  |
| water_weight_Q = calculate_water_weight_Q(...) | Calculates required water weight |
| R = calculate_volume_R(...) | Calculates cement volume |
| V = calculate_air_volume_V(...) | Calculates air volume. |
| X = calculate_total_aggregate_volume_X(...) | Determines total aggregate volume remaining |
| Y = calculate_fine_aggregate_Y(...) | Computes fine aggregate weight |
| Output Section |  |
| print(" NDOT Concrete Mix Design – Weight Summary") | Prints report header |
| print(f"Cement: {cement_A:8.1f} lb") | Displays formatted cement weight. |
| {value:8.1f} | Formats number to 1 decimal place with aligned spacing. |
| This produces a clean summary table of: -Cementitious materials -Aggregates -Water |  |